|  | CrystaLite, Inc. 3307 Cedar Street Everett, WA 98201 | 1-800-666-6065 www.CrystaLiteInc.com info@crystaliteinc.com | CSI Specification: |
| --- | --- | --- | --- |
|  |  |  | 5830 PVC / Aluminum Venting Frame |

08 62 00 – Unit Skylights

### Job Name:

Description:

### GENERAL

  - SECTION INCLUDES

    - This section covers all work necessary for the execution and completion of skylight(s) as shown on drawings and specified herein.

    - Work includes but is not limited to the following: design, fabrication, glazing, and erection of skylight(s) as required for a complete and watertight installation.

  - REFERENCE

    - American Architectural and Manufacturers Association

      - AAMA/WDMA/CSA/101/I.S. 2/A440-05 North American Fenestration Standard/Specification for windows, doors, and skylights. (includes standard test methods for air infiltration, water penetration, structural loading)

      - AAMA 603.8-92 Pigmented Organic Coating on Extruded Aluminum

      - AAMA 611 - Voluntary Standards for Anodized Architectural Aluminum

      - ASTM A 193 / A 193M - 08b Standard Specification for Alloy-Steel and Stainless Steel Bolting Materials for High Temperature or High Pressure Service and Other Special Purpose Applications\

    - American Standards and Test Methods

      - ASTM B 209 - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

      - ASTM B 221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Profiles

      - ASTM D 1667 - 05 Standard Specification for Flexible Cellular Materials - (Closed-Cell Foam)

    - International Building Code 2006

  - DESIGN REQUIREMENTS

Note: Remove all non pertinent information. Edit as required. Glazing options shown are standard. Numerous glazing configurations are available to meet any specific need; including such to meet LEED credit criteria and other daylighting objectives.

    - Unit skylights are certified by National Accreditation & Management Institute and rated by the National Fenestration Rating Council (NFRC) for thermal performance. VT tested values for domes are not certified due to NFRC limitations.

      - 1" OA IGU, Temp / Lam, Clear / Clear, Cardinal LoE 366(2) and Argon Gas Fill:

        - U-Factor: 0.45

        - SHGC: 0.25

        - VT: 0.55

      - Clear Acrylic Double Dome

        - U-Factor: 0.68

        - SHGC: 0.66

        - VT: 0.67

    - Unit skylights are certified by National Accreditation & Management Institute to North American Fenestration Standard/Specification (NAFS) [AAMA/WDMA/CSA 101/I.S.2/A440] for air and water penetration and structural loading - as required by the International Building Code, section 2405.5 Unit Skylights.

      - Glass glazed skylight(s) are labeled with the following certified product rating:

        - MST: SKG-R15 60” x 50”

        - Design Pressure (Download): 135 psf

        - Negative Design (Uplift): 15 psf

        - Water Resistance: 6.0 psf

      - Highload / Snowload glass glazed skylight(s) are labeled with the following certified product rating:

        - Design Pressure (Download): 560 psf

        - Negative Design (Uplift): 15 psf

        - Water Resistance: 6.0 psf

      - Acrylic Double Dome

        - MST: SKP-C30 50” x 50”

        - Design Pressure (Download): 30 psf

        - Negative Design (Uplift): 30 psf

        - Water Resistance: 6.0 psf

  - SUBMITTALS

    - Submit full scale shop drawings indicating methods of construction, location and spacing of anchorage, joinery, finishes, size, shape, thickness of framing members, relationship to adjoining work and glazing materials used.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver materials to site in manufacturer’s original and unopened packaging with parts manifest.

    - Store on site in a location and manner to avoid damage. Stacking should be done in a manner that will prevent damage. Store material in a clean, dry location away from high traffic areas. Any protection on the skylights during transportation should remain until installed.

    - Keep handling on site to a minimum. Exercise caution to avoid damage to finishes of material.

### PRODUCTS

  - MANUFACTURER

    - Drawings and specifications are based on products manufactured by:

    - CrystaLite, Inc.

    - 3307 Cedar Street

    - Everett, WA 98201

    - 1-800-666-6065

    - www.crystaliteinc.com

    - Substitutions:  Not permitted.

  - MATERIALS

    - Aluminum.

      - Aluminum extruded components shall be alloy 6063-T5 or 6063-T6, of sufficient thickness for this application, and as required per structural calculations; ASTM B 221.

      - Aluminum sheet and plate shall be alloy 5052-H32 per ASTM B 209.

    - Glazing Materials

Note: Remove all non pertinent information. Edit as required.

      - Insulated Glass Units

        - Dual seal silicone insulated glass units. 1" OA IGU, Temp / Lam, Clear / Clear, Cardinal LoE 366(2) and Argon Gas Fill. Glass thickness as required by the span and loads.

        - Certified in accordance to:

          - ASTM E 2188 – Standard Test Method for Insulating Glass Unit Performance

          - ASTM E 2189 – Standard Test Method for Testing Resistance to Fogging in Insulating Glass Units

          - ASTM E 2190 – Standard Specification for Insulating Glass Unit Performance and Evaluation

      - Acrylic sheet. Shall be used in all heat formed dome and pyramid shaped skylights. Color shall be (clear, bronze, grey). Thickness as required by the span and loads. Class CC2 Fire Rating

        - Plaskolite Optix White Acrylic Sheet has a measured haze value greater than 90%.

      - Polycarbonate multi-wall sheet. Color shall be (clear, bronze, white). Thickness as required by the span and loads. Fire Rating per application.

      - Polycarbonate monolithic (extended life) sheet. Color shall be (clear, bronze, opal, other). Thickness as required by the span and loads. Class CC1 Fire Rating.

      - Co-polyester alloy monolithic sheet. Color shall be (clear, opal, other). Thickness as required by the span and loads.

  - FINISHES

    - Class II color anodic finish per AA_M12C22A32/A34 complying with AAMA 611. Mechanical finish non-specular as fabricated. Chemical finish etched medium matte. Anodic coating architectural Class II integrally colored or electrolytically deposited color coating 0.4 mil or thicker

  - FASTENERS

    - Exterior fasteners and fasteners exposed to wet areas in frame shall be 300 series stainless steel per ASTM 193/A 193M, except pop rivets used on glazing cap are aluminum or stainless steel per manufacturer.

    - Dry area fasteners shall be cadmium-plated steel per ASTM F 1135 or stainless steel.

    - All welding shall be by the TIG process. All exposed welds to be finished to match frame color where practical.

  - GLAZING ACCESSORIES

    - Glazing tapes per ASTM D 1667, 2240, 3575. All other gaskets, setting blocks, and other materials used in glazing shall be of a type, quality and compatibility to provide performance of the skylight(s) covered in this section.

    - Silicone sealant per CAN/CGSB 19.13-M87; TT-S-001543A/ASTM C 920, Type S, NS, Class 25 use NT, G, A&O test requirements.

  - ACCESSORIES

Note: Remove all non pertinent information. Edit as required.

    - Operator Options

      - Manual Operator: shall be Truth manual chain drive.

      - Electric Operator: shall be operated by Solar Smart: radio remote operated, battery powered, solar charged, chain drive operator system. Requiring no power draw from the building or power grid.

    - Elite (cable) OR Defense (bars) Fall Protection Safety Grids

      - Safety grid(s) demonstrate performance which exceeds Occupational Safety and Health Administration (OSHA) and the Washington Industrial Safety and Health Act (WISHA) requirements for providing fall protection; static loads of 200 lbs and 800 lbs respectively. Safety grid(s), resisted without failure dynamic impacts of 800 lbs created by a 200 lbs heavy bag released from 48 inches; followed by holding 1,038 lbs of static load for 5 minutes. Testing was witnessed for verification by a third party accredited testing laboratory. Test report available upon request.

    - Self-flashing Insulated Aluminum Curb

      - Aluminum sheet and plate shall be alloy 5052-H32 per ASTM B 209.

      - Curb shall be insulated with Thermasheath rigid foam plastic thermal insulation board composed of polyisocyanurate foam core bonded to reinforced aluminum foil facers on each side. Thickness shall be 1" nominal providing 5.9 R-Value determined by ASTM C518.

  - FABRICATION

    - Skylight(s) shall be factory fabricated and preassembled in largest size assemblies possible with considerations for shipping and jobsite handling.

    - Skylight(s) shall have properly designed weep systems for drainage to exterior.

### EXECUTION

  - INSTALLATION

    - Framing shall be installed and glazed by experienced workmen in accordance with the approved shop drawings, manufacturer’s instructions and glazing standards.

  - CLEANING AND PROTECTION

    - Subsequent to installation of skylight(s) the General Contractor shall be responsible for the cleanup and protection of all materials provided per this section, including, but not limited to glazing materials and framing members. No abrasive materials of any kind shall be used in cleaning of skylight surfaces.