|  | CrystaLite, Inc. 3307 Cedar Street Everett, WA 98201 | 1-800-666-6065 www.CrystaLiteInc.com info@crystaliteinc.com | CSI Specification: |
| --- | --- | --- | --- |
|  |  |  | 5842 PVC / Aluminum Frame |

08 62 00 – Unit Skylights

### Job Name:

Description:

### GENERAL

  - SECTION INCLUDES

    - This section covers all work necessary for the execution and completion of skylight(s) as shown on drawings and specified herein.

    - Work includes but is not limited to the following: design, fabrication, glazing, and erection of skylight(s) as required for a complete and watertight installation.

  - REFERENCE

    - American Architectural and Manufacturers Association

      - AAMA/WDMA/CSA/101/I.S. 2/A440-05 North American Fenestration Standard/Specification for windows, doors, and skylights. (includes standard test methods for air infiltration, water penetration, structural loading)

      - AAMA 603.8-92 Pigmented Organic Coating on Extruded Aluminum

      - AAMA 611 - Voluntary Standards for Anodized Architectural Aluminum

      - ASTM A 193 / A 193M - 08b Standard Specification for Alloy-Steel and Stainless Steel Bolting Materials for High Temperature or High Pressure Service and Other Special Purpose Applications\

    - American Standards and Test Methods

      - ASTM B 209 - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

      - ASTM B 221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Profiles

      - ASTM D 1667 - 05 Standard Specification for Flexible Cellular Materials - (Closed-Cell Foam)

    - International Building Code

    - Unified Facilities Criteria (UFC)

      - DoD Minimum Antiterrorism Standards for Buildings

  - DESIGN REQUIREMENTS

Note: Remove all non pertinent information. Edit as required. Glazing options shown are standard. Numerous glazing configurations are available to meet any specific need; including such to meet LEED credit criteria and other daylighting objectives.

    - Unit skylights are certified by National Accreditation & Management Institute and rated by the National Fenestration Rating Council (NFRC) for thermal performance. VT tested values for domes are not certified due to NFRC limitations.

      - 1" OA IGU, Temp / Lam, Clear / Clear, Cardinal LoE 366(2) and Argon Gas Fill:

        - U-Factor: 0.50

        - SHGC: 0.27

        - VT: 0.63

      - Clear Acrylic Double Dome

          - U-Factor: 0.68

          - SHGC: 0.75

          - VT: 0.73

      - Clear Acrylic Triple Dome

          - U-Factor: 0.50

          - SHGC: 0.57

          - VT: 0.63

    - Unit skylights are certified by National Accreditation & Management Institute to North American Fenestration Standard/Specification (NAFS) [AAMA/WDMA/CSA 101/I.S.2/A440] for air and water penetration and structural loading - as required by the International Building Code, section 2405.5 Unit Skylights.

      - Glass glazed skylight(s) are labeled with the following certified product rating:

        - MST: SKG-C30 74” x 50”

        - Design Pressure (Download): 85 psf

        - Negative Design (Uplift): 30 psf

        - Water Resistance: 6.0 psf

      - Highload / Snowload glass glazed skylight(s) are labeled with the following certified product rating:

        - Design Pressure (Download): 520 psf

        - Negative Design (Uplift): 30 psf

        - Water Resistance: 6.0 psf

      - Acrylic Dome

        - MST: SKP-C30 50” x 50”

        - Design Pressure (Download): 45 psf

        - Negative Design (Uplift): 30 psf

        - Water Resistance: 6.0 psf

Note: Remove all non pertinent information. Edit as required.

    - Fall Protection

      - Skylight glazing shall demonstrate performance which exceeds Occupational Safety and Health Administration (OSHA) requirements for providing fall protection by the application of dynamic loads of a 200 pound impact bag dropped from a height of 6’ twice and 7’ once, thus applying dynamic loads of 1,200 and 1,400 ft-lbs of force respectively, all on the same test specimen; remaining undamaged after impacts.

  - SUBMITTALS

    - Submit full scale shop drawings indicating methods of construction, location and spacing of anchorage, joinery, finishes, size, shape, thickness of framing members, relationship to adjoining work and glazing materials used.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver materials to site in manufacturer’s original and unopened packaging with parts manifest.

    - Store on site in a location and manner to avoid damage. Stacking should be done in a manner that will prevent damage. Store material in a clean, dry location away from high traffic areas. Any protection on the skylights during transportation should remain until installed.

    - Keep handling on site to a minimum. Exercise caution to avoid damage to finishes of material.

### PRODUCTS

  - MANUFACTURER

    - Drawings and specifications are based on products manufactured by:

    - CrystaLite, Inc.

    - 3307 Cedar Street

    - Everett, WA 98201

    - 1-800-666-6065

    - www.crystaliteinc.com

    - Substitutions:  Not permitted.

  - MATERIALS

    - Aluminum.

      - Aluminum extruded components shall be alloy 6063-T5 or 6063-T6, of sufficient thickness for this application, and as required per structural calculations; ASTM B 221.

      - Aluminum sheet and plate shall be alloy 5052-H32 per ASTM B 209.

    - Glazing Materials

Note: Remove all non pertinent information. Edit as required.

      - Insulated Glass Units

        - Dual seal silicone insulated glass units. 1" OA IGU, Temp / Lam, Clear / Clear, Cardinal LoE 366(2) and Argon Gas Fill. Glass thickness as required by the span and loads.

          - Laminated glass shall include a minimum interlayer thickness of 0.030"

            - Compliant to IBC 2405.2 and 2405.3 Screening.

            - Compliant to UFC 4-010-01 criteria for Conventional Construction Standoff Distance per ASTM F 2248

        - Saflex white laminate has a measured haze value greater than 90% per ASTM D1003.

        - Certified in accordance to:

          - ASTM E 2188 – Standard Test Method for Insulating Glass Unit Performance

          - ASTM E 2189 – Standard Test Method for Testing Resistance to Fogging in Insulating Glass Units

          - ASTM E 2190 – Standard Specification for Insulating Glass Unit Performance and Evaluation

      - Impact Modified Acrylic Plastic, shall meet or exceed the following:

        - ICC-ES listings as CC2 approved plastic complying with Section 2606.4 of the IBC

        - ANSI Z97.1 Safety Glazing Requirements

        - Underwriters Laboratories (UL) listings as UL 94HB and UL746 Compliant

        - Miami-Dade County Product Notice of Acceptance-Plastics

        - ASTM D-4802 B-1 Plastics

        - Plaskolite Optix White Acrylic Sheet has a measured haze value greater than 90%.

      - Polycarbonate multi-wall sheet. Thickness as required by the span and loads.

        - Fire rating: Class A per ASTM E-84. Class CCI per ASTM D-635. Maximum Flame Spread/Smoke Development: 0/85

        - Impact resistance, when tested in accordance with ASTM D 5420 (Drop Dart): >72 foot-pounds (271.16 N m) for 1/4-inch (6 mm) thick material.

        - Weather resistance, when tested for 1500 hours in accordance with ÅSTM G 53 and QUV 313B:

        - Coating integrity: Intact after testing period.

        - Compliant to IBC 2405.2 and 2405.3 Screening.

        - Compliant to UFC 4-010-01 criteria for Conventional Construction Standoff Distance per ASTM F 2248

      - Polycarbonate monolithic (extended life) sheet. Thickness as required by the span and loads.

        - Class CC1 Fire Rating.

        - Compliant to IBC 2405.2 and 2405.3 Screening.

        - Compliant to UFC 4-010-01 criteria for Conventional Construction Standoff Distance per ASTM F 2248

      - Co-polyester alloy monolithic sheet, shall meet or exceed the following:

        - Rate of Burn, per ASTM D 635: CC1 classification

        - ICC-ES listings as CC1 approved plastic complying with Section 2606.4 of the IBC

        - ANSI Z97.1 Safety Glazing Requirements

        - Miami-Dade County Product Notice of Acceptance-Plastics

        - Light-transmitting plastic in accordance with Section 2608 of the IBC

        - Weathering data in accordance to ICC-ES AC16

        - Compliant to UFC 4-010-01 criteria for Conventional Construction Standoff Distance per ASTM F 2248

  - FINISHES

    - Class II color anodic finish per AA_M12C22A32/A34 complying with AAMA 611. Mechanical finish non-specular as fabricated. Chemical finish etched medium matte. Anodic coating architectural Class II integrally colored or electrolytically deposited color coating 0.4 mil or thicker

  - FASTENERS

    - Exterior fasteners and fasteners exposed to wet areas in frame shall be 300 series stainless steel per ASTM 193/A 193M, except pop rivets used on glazing cap are aluminum or stainless steel per manufacturer.

    - Dry area fasteners shall be cadmium-plated steel per ASTM F 1135 or stainless steel.

    - All welding shall be by the TIG process. All exposed welds to be finished to match frame color where practical.

  - GLAZING ACCESSORIES

    - Glazing tapes per ASTM D 1667, 2240, 3575. All other gaskets, setting blocks, and other materials used in glazing shall be of a type, quality and compatibility to provide performance of the skylight(s) covered in this section.

    - Silicone sealant per CAN/CGSB 19.13-M87; TT-S-001543A/ASTM C 920, Type S, NS, Class 25 use NT, G, A&O test requirements.

  - ACCESSORIES

Note: Remove all non pertinent information. Edit as required.

    - Elite (cable) OR Defense (bars) Fall Protection Safety Grids

      - Safety grid(s) demonstrate performance which exceeds Occupational Safety and Health Administration (OSHA) and the Washington Industrial Safety and Health Act (WISHA) requirements for providing fall protection; static loads of 200 lbs and 800 lbs respectively. Safety grid(s), resisted without failure dynamic impacts of 800 lbs created by a 200 lbs heavy bag released from 48 inches; followed by holding 1,038 lbs of static load for 5 minutes. Testing was witnessed for verification by a third party accredited testing laboratory. Test report available upon request.

    - Self-flashing Insulated Aluminum Curb

      - Aluminum sheet and plate shall be alloy 5052-H32 per ASTM B 209.

      - Curb shall be insulated with Thermasheath rigid foam plastic thermal insulation board composed of polyisocyanurate foam core bonded to reinforced aluminum foil facers on each side. Thickness shall be 1" nominal providing 5.9 R-Value determined by ASTM C518.

  - FABRICATION

    - Skylight(s) shall be factory fabricated and preassembled in largest size assemblies possible with considerations for shipping and jobsite handling.

    - Skylight(s) shall have properly designed weep systems for drainage to exterior.

### EXECUTION

  - INSTALLATION

    - Framing shall be installed and glazed by experienced workmen in accordance with the approved shop drawings, manufacturer’s instructions and glazing standards.

  - CLEANING AND PROTECTION

    - Subsequent to installation of skylight(s) the General Contractor shall be responsible for the cleanup and protection of all materials provided per this section, including, but not limited to glazing materials and framing members. No abrasive materials of any kind shall be used in cleaning of skylight surfaces.

  - WARRANTY

    - Glass Glazed Skylights

      - CrystaLite glass skylights are warranted against manufacturer’s defects for a period of ten (10) years from date of purchase. Insulated glass units shall carry a ten (10) year warranty against delaminating, seal failure and deterioration of low-e coatings. Where field glazed, the installer shall be responsible for glass to frame seal.

      - A minimum pitch of 2/12 is required for all glass skylights. Any modification and/ or changes to our skylight system without the written consent of CrystaLite, Inc. will render the warranty null and void.

    - Plastic Glazed Skylights

      - CrystaLite thermoformed plastic skylights are warranted against manufacturer’s defects for a period of five (5) years from date of purchase.

      - CrystaLite skylights glazed with Lexan polycarbonate multiwall sheet shall have a ten (10) year prorated warranty from date of purchase.

    - Stipulations

      - Warranty replacement units will be remanufactured in the same size and description as the original units only and does not include removal of defective unit or installation of replacement unit. Warranty period will remain in effect from the original date of delivery. Warranty orders will be chargeable items with proper credit given upon return and inspection of original item.

    - Operator Hardware

      - Solar Smart products are warranted against defect in materials and workmanship for a period of two (2) years from date of purchase.

      - All Truth Hardware products, with the exception of electrical products, are warranted against defect in materials and workmanship for a period of ten (10) years from date of purchase. All Truth Hardware electrical products are warranted for one (1) year against defects in materials and workmanship.